# USB Type-C Interface

This circuit provides a minimal USB Type-C interface for ESP32-based boards, enabling 5 V power input and USB 2.0 data communication.

USB Power Delivery (USB PD) is not supported. The circuit is designed for use with a standard USB Type-C cable, providing 5 V power and USB-to-Serial connectivity via interfaces such as the CH340.

## Circuit Overview

### 1. USB Type-C Connector (J1)

Component

TYPE-C-31-M-12

Why This Component Was Chosen

- Low-cost and widely available USB Type-C connector

- Suitable for most ESP32 boards that use only USB 2.0

- Supports cable insertion in either orientation

- Ideal for simple 5 V devices that do not require USB Power Delivery (PD)

Pins Used

| Pin | Function |
| --- | --- |
| VBUS | +5 V Input |
| GND | Ground |
| DP1 / DP2 | USB D+ |
| DN1 / DN2 | USB D− |
| CC1 / CC2 | USB Configuration Channel |
| Shield | Connector Shield |

### 2. CC Resistors (R11, R12)

Component

5.1 kΩ

Why This Component Was Chosen

According to the USB Type-C specification, a power-consuming device (UFP, Upstream Facing Port) must connect a 5.1 kΩ pull-down (Rd) resistor to both the CC1 and CC2 pins.

These resistors allow the USB host to recognize that:

"This is a power-consuming device."

Since the USB Type-C cable can be inserted in either orientation, identical resistors are connected to both CC1 and CC2.

The connections are as follows:

CC1 → 5.1 kΩ → GND

CC2 → 5.1 kΩ → GND

### 3. USB Data Line Connections

Because USB Type-C supports reversible plug insertion, the connector contains two pairs of USB data pins:

DP1

DP2

DN1

DN2

The circuit connects them as follows:

DP1 + DP2 → USB_DP

DN1 + DN2 → USB_DN

This arrangement ensures that the same USB signals are available regardless of the cable orientation.

It is the most commonly used wiring method for USB 2.0 Type-C interfaces.

### 4. VBUS

VBUS is the +5 V power supplied by the USB host.

This power is typically connected to:

- 3.3 V regulator

- Battery charging IC

- Power source selection circuit

### 5. C1 (100 nF)

Component

100 nF Ceramic Capacitor

Why This Component Was Chosen

Placed between VBUS and GND, this capacitor acts as a decoupling capacitor to:

- Filter high-frequency noise

- Reduce transient voltage fluctuations

- Improve power stability during USB connection

For the best performance, this capacitor should be placed as close to the USB connector as possible.

### 6. ESD Protection (D1, D2, D3)

Component

LESD5D5.0CT1G

Why This Component Was Chosen

Because the USB connector is directly exposed to the outside environment, it is highly susceptible to electrostatic discharge (ESD).

These TVS diodes protect the circuit against:

- IEC 61000-4-2 ESD events

- Transient surges generated during cable insertion

- Static electricity from user contact

In this design, protection is provided for:

- D+

- D−

- VBUS

For maximum effectiveness, the ESD protection devices should be placed as close to the USB connector as possible.

## Design Goals

This project was designed as a reusable USB Type-C input circuit that is simple enough for beginners to understand while remaining suitable for use in their own projects.

The following design principles were applied:

- Simplified the circuit by excluding USB Power Delivery (PD)

- Optimized for ESP32 projects that require only USB 2.0

- Supports reversible USB Type-C cable insertion

- Uses standard 5.1 kΩ CC pull-down resistors

- Includes ESD protection circuitry

- Achieves broad compatibility with a minimal number of components

This circuit is a general-purpose USB Type-C interface that can be reused not only with ESP32, but also with most USB 2.0-based microcontroller boards, including STM32, RP2040, AVR, CH340, CP2102, and FT232 platforms.

Regulator

This circuit is a power regulator designed to convert the +5 V supply provided by the USB Type-C connector into the +3.3 V supply required by the ESP32 and its peripheral circuits.

Its simple design follows a standard LDO (Low Dropout Regulator) configuration that is widely used in a variety of electronic projects.

## Circuit Overview

### 1. LDO Regulator (U1)

Component

AMS1117-3.3

Why This Component Was Chosen

The AMS1117-3.3 is one of the most widely used 3.3 V linear regulators (LDOs).

It was selected for the following reasons:

- Very low cost

- Supported by the SMT assembly services of most PCB manufacturers

- Readily available

- Requires only a minimal number of external components

- A proven device widely used on ESP32 development boards

The regulator accepts the USB +5 V supply as its input and provides a stable +3.3 V output.

The pin configuration is as follows.

| Pin | Function |
| --- | --- |
| VI | +5 V Input |
| GND | Ground |
| VO | +3.3 V Output |

### 2. Input Capacitor (C2)

Component

10 μF Ceramic Capacitor (0402)

Why This Component Was Chosen

When a USB cable is connected or the load changes, transient noise and voltage drops may occur on the input supply.

The input capacitor provides the following functions:

- Stabilizes the input voltage

- Supplies transient current during load changes

- Filters high-frequency noise

For best performance, the capacitor should be placed as close as possible to the regulator's input pin.

### 3. Output Capacitor (C3)

Component

10 μF Ceramic Capacitor (0402)

Why This Component Was Chosen

An output capacitor is essential for the stable operation of the regulator.

Its primary functions are:

- Stabilizes the output voltage

- Improves transient response to load changes

- Reduces output voltage ripple

- Prevents oscillation

The output capacitor should also be placed as close as possible to the regulator's output pin.

## Design Considerations When Using the AMS1117

Since the AMS1117 is a linear regulator (LDO), the voltage difference between the input and output is dissipated as heat.

For example:

Input Voltage: 5 V

Output Voltage: 3.3 V

Output Current: 300 mA

The resulting power dissipation is:

P = (5 V − 3.3 V) × 0.3 A = 0.51 W

As the output current increases, heat generation also increases. Therefore, for applications that continuously draw high current, it is recommended to provide sufficient PCB ground copper area to improve heat dissipation.

The AMS1117 performs reliably in most general-purpose ESP32 applications. However, because of its relatively low efficiency, it is not the best choice for battery-powered, low-power devices where maximizing battery life is important.

## Design Goals

The objective of this project is to provide a versatile 3.3 V power supply circuit using components that are inexpensive, readily available, and widely proven in a variety of electronic projects.

The following design principles were applied:

- Uses the low-cost and readily available AMS1117 regulator

- Requires only a minimal number of external components

- Converts USB +5 V into a stable +3.3 V supply

- Features a simple circuit that is easy for beginners to understand

- Can be reused in most 3.3 V microcontroller projects, including ESP32, STM32, RP2040, and AVR platforms

For future low-power or battery-operated designs, replacing the AMS1117 with a high-efficiency buck converter or a low quiescent current (Low-IQ) LDO can significantly improve battery life.

Firmware Download

This circuit is designed to download firmware to the ESP32 and provide serial communication using the CH340C USB-to-UART bridge.

It also uses the DTR and RTS control signals to automatically enter Download Mode, allowing firmware to be uploaded directly from the Arduino IDE or ESP-IDF without the need to manually press any buttons.

## Circuit Overview

### 1. USB-UART Bridge (U2)

Component

CH340C

Why This Component Was Chosen

The CH340C is one of the most widely used USB-to-UART bridge ICs.

It was selected for the following reasons:

- Very low cost

- Supported by JLCPCB SMT assembly services

- Does not require an external crystal oscillator

- Driver support is available for most operating systems

- One of the most commonly used USB-to-UART ICs on ESP32 development boards

Its primary functions include:

- USB-to-UART conversion

- Firmware download

- Serial Monitor communication

- Automatic reset support

### 2. UART Signals

The CH340C communicates with the ESP32 through a UART interface.

The connections are as follows:

TXD → ESP32 RXD0

RXD → ESP32 TXD0

ESP32 UART0 is the primary UART used by both the bootloader and the Serial Monitor.

### 3. 0 Ω Jumper Resistors (R3, R4)

Component

0 Ω Resistor (0402)

Why This Component Was Chosen

0 Ω resistors are inserted in the UART signal lines.

This provides several advantages:

- Simplifies future PCB modifications

- Allows the UART lines to be easily disconnected if necessary

- Makes oscilloscope measurements more convenient

- Simplifies replacement with a different UART device

In production, these resistors simply function as jumpers.

### 4. Auto Download Circuit

To enter Download Mode, the ESP32 requires the following two pins to be controlled in a specific sequence:

EN

IO0

This circuit uses the CH340C control signals:

DTR

RTS

Two NPN transistors (SS8050) automatically control these signals as follows:

DTR → EN

RTS → IO0

When a firmware download is started from the Arduino IDE or ESP-IDF, the following sequence is performed automatically:

- EN Reset

- IO0 Low

- Enter Download Mode

As a result, there is no need to manually press either the Reset or Boot button.

### 5. Base Resistors (R5, R6)

Component

10 kΩ (0402)

Why This Component Was Chosen

These resistors limit the base current of the transistors.

Their functions include:

- Limiting base current

- Protecting the CH340C outputs

- Ensuring reliable transistor switching

A value of 10 kΩ is the most commonly used for this application.

### 6. Auto Reset Transistors (Q1, Q2)

Component

SS8050

Why This Component Was Chosen

The SS8050 is a general-purpose NPN transistor.

It was selected for the following reasons:

- Very low cost

- Readily available

- Adequate switching speed

- Commonly used in ESP32 Auto Download circuits

In this circuit, the transistors function solely as electronic switches.

### 7. IO0 Capacitor (C6)

Component

100 nF Ceramic Capacitor

Why This Component Was Chosen

This capacitor reduces noise on the IO0 line, making entry into Download Mode more reliable.

It also helps prevent unintended operation caused by transient signal fluctuations.

## Design Goals

This project was designed to provide a USB-to-UART download circuit that delivers the same user experience as a typical ESP32 development board.

The following design principles were applied:

- Uses the CH340C, which does not require an external crystal oscillator

- Supports automatic firmware download

- Supports automatic reset

- Requires only a minimal number of external components

- Improves maintainability by including 0 Ω jumpers on the UART lines

- Provides a simple, reusable circuit that is easy for beginners to understand

This circuit can be used almost identically with most ESP32-family MCUs, including the ESP32-WROOM, ESP32-S3, and ESP32-C3. It can also be directly applied to the design of USB-to-UART-based development boards and IoT devices.

Buttons

This circuit provides a Reset button and a User Button for basic control of the ESP32.

Both buttons use an active-low configuration. Under normal conditions, the signals are held at a logic High level by pull-up resistors. When a button is pressed, the corresponding signal is connected to GND, producing a logic Low input.

The circuit also incorporates RC filtering to reduce noise and ensure more stable and reliable button operation.

## Circuit Overview

### 1. Reset Button (S3)

Component

SKPMAME010 Tactile Switch

Why This Component Was Chosen

The Reset button directly controls the ESP32 EN (Enable) pin.

When the button is pressed, the EN pin is pulled Low, resetting the ESP32. Releasing the button returns the EN pin to High, allowing the ESP32 to boot normally.

This is the standard reset configuration used on most ESP32 development boards.

### 2. User Button (S2)

Component

SKPMAME010 Tactile Switch

Why This Component Was Chosen

The User Button is connected to ESP32 GPIO34.

Since GPIO34 is an input-only pin, it is well suited for use as a user input button.

Depending on the application, the button can be assigned to functions such as:

- User input

- Menu selection

- Configuration changes

- Start BLE pairing

- Reset Wi-Fi settings

- Factory reset

- Any other user-defined function

Its function can be freely customized to suit the requirements of the project.

### 3. Pull-up Resistors (R1, R2)

Component

10 kΩ (0402)

Why This Component Was Chosen

The ESP32 EN and GPIO34 pins must normally remain at a logic High level.

To achieve this, each pin is connected to a 10 kΩ pull-up resistor.

Normal state:

+3.3 V

↓

10 kΩ

↓

GPIO

When the button is pressed:

GPIO

↓

Button

↓

GND

The GPIO is therefore pulled Low.

A value of 10 kΩ is the most commonly used pull-up resistance because it provides an excellent balance between low power consumption and good noise immunity.

### 4. Noise Filtering Capacitors (C4, C5)

Component

100 nF Ceramic Capacitor

Why This Component Was Chosen

Mechanical push buttons produce contact bounce, where the electrical contacts rapidly make and break connection when the button is pressed or released.

The 100 nF capacitor helps to:

- Reduce button bounce

- Filter transient noise

- Improve reset reliability

- Prevent unintended GPIO triggering

In particular, the capacitor in the reset circuit helps maintain a stable EN signal during power-up.

## Button Design Considerations

The ESP32 EN pin is an active-low input.

Pressing the Reset button immediately resets the ESP32, and releasing it allows the device to boot normally.

GPIO34 is also used as an active-low input. Therefore, the firmware typically interprets the button state as follows:

- Button released → Logic High

- Button pressed → Logic Low

To compensate for mechanical switch bounce, it is recommended to implement software debouncing with a delay of approximately 10–30 ms.

## Design Goals

This project adopts the same button circuitry found on most ESP32 development boards, making it easy for beginners to understand and reuse in their own designs.

The following design principles were applied:

- Uses a standard active-low button configuration

- Maintains a stable default state with 10 kΩ pull-up resistors

- Reduces noise and switch bounce using 100 nF capacitors

- Implements a standard reset circuit using the ESP32 EN pin

- Provides a general-purpose user input button using GPIO34

- Offers a reusable design suitable for a wide range of ESP32 projects

This circuit can be used not only with the ESP32, but also with most other microcontrollers, including STM32, RP2040, and AVR devices. It serves as a practical, reusable template for implementing both button input and reset circuitry.

LEDs

This circuit provides both a Power LED and a User LED to indicate the operating status of the board.

The Power LED indicates that the board is receiving power correctly, while the User LED can be freely controlled by the firmware to provide visual feedback for various operating states.

Both LEDs are configured as active-low, allowing them to be driven easily by the ESP32.

Circuit Overview

- Power LED (LED1)

Component

KT-0805G (Green LED)

Why This Component Was Chosen

The Power LED indicates whether the +3.3 V supply is being delivered to the board correctly.

It remains illuminated whenever power is present and can be used for the following purposes.

• Confirming that power is being supplied
• Checking for power-related faults
• Monitoring the power status during debugging

Because it allows the power status to be checked immediately, it is very useful during development and maintenance.

- User LED (LED2)

Component

NCD0805R1 (Red LED)

Why This Component Was Chosen

The User LED is connected to ESP32 GPIO18 and can be freely controlled by the firmware.

It can be used for the following purposes.

• Indicating that the boot process is complete
• Indicating BLE advertising status
• Indicating Wi-Fi connection status
• Indicating data transmission and reception
• Indicating errors
• Indicating firmware update status
• Indicating user-defined operating states

It can be assigned to various functions depending on the purpose of the project.

- Current-Limiting Resistors (R7, R8)

Component

10 kΩ (0402)

Why This Component Was Chosen

LEDs must always be used with current-limiting resistors.

In this circuit, 10 kΩ resistors are used to significantly reduce the LED brightness.

This value was selected for the following reasons.

• Reduces unnecessary power consumption
• Extends battery operating time
• Reduces glare in indoor environments
• Provides sufficient brightness for status indication

General-purpose development boards commonly use resistor values between 330 Ω and 1 kΩ. However, status LEDs that remain continuously illuminated do not need to be excessively bright.

In battery-powered IoT devices, even a difference of several hundred microamperes can affect battery life over extended periods. Therefore, higher resistance values are used in this design to reduce power consumption.

LED Operation

Power LED

The Power LED is connected between +3.3 V and GND.

It remains illuminated whenever power is supplied.

+3.3 V
↓
R7
↓
LED
↓
GND

User LED

The User LED is connected between +3.3 V and GPIO18.

When GPIO18 is driven Low, current flows through the LED and turns it on.

+3.3 V
↓
R8
↓
LED
↓
GPIO18
↓
GND

Therefore,

GPIO18 = Low → LED ON

GPIO18 = High → LED OFF

The User LED operates in an active-low configuration.

This is one of the most widely used LED connection methods on ESP32 development boards.

Design Goals

This project was designed to provide a status indication circuit suitable for low-power IoT devices.

The following design principles were applied.

• Provides a Power LED for immediate confirmation of the power status
• Provides a general-purpose User LED controlled through a GPIO pin
• Uses high resistor values to minimize power consumption
• Uses an active-low configuration consistent with common ESP32 development boards
• Provides a simple circuit that beginners can easily understand and reuse

This circuit can be used not only with the ESP32, but also with most microcontroller platforms, including STM32, RP2040, and AVR. It is a general-purpose LED circuit suitable for both development boards and battery-powered IoT products.

ESP32-WROOM-32E Module

This circuit implements the minimum system required for an ESP32-WROOM-32E module.

It includes the essential power decoupling circuitry and the basic boot configuration required for reliable operation. In addition, all major ESP32 GPIO pins are brought out to external connectors, making it easy to interface with a wide variety of sensors and peripheral devices.

The design is intended to serve as a general-purpose template that can be used as the starting point for new ESP32-based projects.

## Circuit Overview

### 1. ESP32-WROOM-32E Module (U5)

Component

ESP32-WROOM-32E

Why This Component Was Chosen

The ESP32-WROOM-32E is Espressif's flagship Wi-Fi and Bluetooth (BLE) module.

It was selected for the following reasons:

- Built-in Wi-Fi and Bluetooth Low Energy (BLE)

- Large number of available GPIO pins

- Supported by both Arduino IDE and ESP-IDF

- Extensive software libraries and example projects

- One of the most widely used ESP32 modules worldwide

- Supported by JLCPCB SMT assembly services

In this project, most of the ESP32 GPIO pins are brought out to external connectors, making the board suitable for a wide variety of applications.

### 2. Power Decoupling Capacitors (C7, C8)

Components

- C7: 100 nF Ceramic Capacitor (0402)

- C8: 22 μF Ceramic Capacitor (0805)

Why These Components Were Chosen

The ESP32 can draw instantaneous currents of several hundred milliamps during Wi-Fi transmission.

To ensure a stable power supply, two different capacitors are used.

C7 (100 nF)

A high-frequency decoupling capacitor.

Placed close to the ESP32 VDD pins, it suppresses high-frequency switching noise generated inside the module.

C8 (22 μF)

A bulk capacitor that supplies transient current during sudden increases in power demand.

It helps minimize voltage drops during Wi-Fi or Bluetooth transmissions, ensuring stable operation of the ESP32.

### 3. IO0 Jumper Resistor (R10)

Component

0 Ω Resistor (0402)

Why This Component Was Chosen

IO0 is an important pin that determines the ESP32 boot mode.

Adding a 0 Ω resistor provides several advantages:

- Allows the IO0 circuit to be modified easily

- Simplifies circuit debugging

- Makes it easy to isolate the GPIO function from the boot function when required

- Improves accessibility for measurements and prototyping

## Breakout Connectors

Most of the ESP32 GPIO pins are routed to external connectors (J2 and J3), allowing expansion modules and peripheral devices to be connected easily.

### 4. Breakout Connectors (J2, J3)

Component

Conn_01x17

Why This Component Was Chosen

Most of the ESP32 GPIO pins are brought out to external headers for:

- Sensor connections

- Actuator connections

- Debugging

- Prototyping

- Expansion board connections

This arrangement is particularly convenient when using breadboards or jumper wires during development.

### 5. Power Pins

The breakout connectors also provide the following power connections.

+5 V

Power supplied directly from the USB connector.

+3.3 V

Regulated power supplied by the onboard voltage regulator.

GND

Multiple ground pins are provided to simplify connections to sensors and peripheral modules.

### 6. Dedicated I²C Pins

The following pins are assigned as the default I²C interface.

GPIO21 (SDA)

GPIO22 (SCL)

These assignments match the default configuration of the ESP32 Wire library, allowing most I²C sensors to be connected without additional software configuration.

### 7. ADC Inputs

The following pins are arranged for convenient analog input connections.

SENSOR_VP

SENSOR_VN

GPIO34

GPIO35

These pins are well suited for sensor inputs, analog signal acquisition, and voltage measurement applications.

## Power Bypass Capacitors

### C9

Component

100 nF Ceramic Capacitor

Placed near the +3.3 V supply on the breakout connector, this capacitor reduces noise that may be introduced by externally connected sensors and peripheral modules.

### C10

Component

100 nF Ceramic Capacitor

A bypass capacitor for the +5 V power rail.

It filters high-frequency noise from the USB power supply, providing a cleaner and more stable supply for external devices.

### C11

Component

100 nF Ceramic Capacitor

A decoupling capacitor for the +3.3 V supply located on the opposite side of the breakout connectors.

It helps suppress noise generated by externally connected sensors and modules.

## Design Considerations When Using the ESP32

Some ESP32 GPIO pins serve special functions during the boot process as boot strapping pins.

Examples include:

- GPIO0

- GPIO2

- GPIO5

- GPIO12

- GPIO15

These pins can affect the boot mode of the ESP32 and should therefore be used with care when connecting external circuitry.

In addition, GPIO34 through GPIO39 are input-only pins, which means they cannot be used to drive output devices such as LEDs or relays.

Considering these GPIO characteristics during the design phase can help prevent unexpected boot issues and hardware conflicts.

## Design Goals

This project was designed as a general-purpose minimum system that can serve as the starting point for most ESP32-based IoT and embedded applications.

The following design principles were applied:

- Implements the standard minimum system for the ESP32-WROOM-32E

- Includes a stable power supply suitable for Wi-Fi operation

- Brings all major GPIO pins out to breakout connectors

- Provides convenient access to I²C and ADC pins

- Uses sufficient decoupling capacitors to ensure power stability

- Features a simple, reusable design that is easy for beginners to understand

This circuit serves as a versatile hardware template for a wide range of ESP32-based projects, including IoT devices, sensor nodes, data loggers, environmental monitoring systems, and Bluetooth Low Energy (BLE) applications.