Component Replacement Guide

This document explains how to safely replace components in the KiCad project when the original parts are out of stock or need to be substituted with equivalent components from other manufacturers.

When replacing a component, do not rely solely on its name or physical appearance. Always verify its electrical characteristics, pin configuration, package type, footprint, voltage and current ratings, and other critical specifications.

For components such as USB Type-C connectors, switches, voltage regulators, and USB-to-UART bridge ICs, the pin assignments and mechanical dimensions often vary between manufacturers. Therefore, always consult the component's datasheet before making a replacement.

1. Things to Check Before Replacing a Component

Before selecting a replacement component, verify the following items.

Electrical Characteristics

- Operating voltage

- Maximum input voltage

- Output voltage

- Maximum current rating

- Current consumption

- Signal speed

- Polarity

- Protection features

Pin Configuration

Even components with the same function may have different pin numbers or pin assignments.

For example, a 3.3 V voltage regulator may have different pin orders depending on the manufacturer or package type, such as:

- Input

- Ground

- Output

- Enable

Before reusing the existing KiCad symbol, verify that the pin numbers in the symbol match those specified in the datasheet of the replacement component.

Package and Footprint

Check the following items:

- SMD or through-hole package

- Package type

- Body dimensions

- Pad spacing

- Number of pins

- Pin numbering orientation

- Mounting height

- Recommended land pattern

Even if two components share the same package name, the recommended PCB land pattern may differ between manufacturers.

Electrical Ratings

The replacement component should have ratings that are equal to or greater than those of the original component.

For example, if the original capacitor is rated at 10 μF, 10 V, the replacement should also be rated for at least 10 μF and 10 V, or higher. 2. Basic Procedure for Replacing a Component in KiCad

- Obtain the datasheet for the replacement component.

- Compare the electrical characteristics of the original and replacement components.

- Verify the pin configuration of the replacement component.

- Check whether a suitable symbol and footprint are available in the KiCad standard libraries.

- If no suitable symbol exists, create a new symbol.

- If no suitable footprint exists, create a new footprint.

- Replace the symbol or update the component properties in the schematic.

- Verify that the pin numbers and schematic connections match the new symbol.

- Update the PCB layout from the modified schematic.

- Verify the position and orientation of the new footprint.

- Modify the PCB routing as required.

- Run ERC (Electrical Rules Check) and DRC (Design Rules Check).

- Use the 3D Viewer to verify the component orientation and check for mechanical interference.

- Before manufacturing, perform a final review of the Gerber files and assembly files.

3. Replacing the USB Type-C Connector

Original Component

TYPE-C-31-M-12

Possible Replacements

- HRO TYPE-C-31-M-12

- GCT USB Type-C Connector

- Amphenol USB Type-C Connector

- USB 2.0 Type-C connectors from other manufacturers

4. Replacing the USB-to-UART Bridge IC

Original Component

CH340C

Possible Replacements

- CH340N

- CH9102F

- CP2102N

- FT231X

- Other USB-to-UART bridge ICs

5. Replacing the 3.3 V Voltage Regulator

Original Component

AMS1117-3.3

Possible Replacements

- AP2112K-3.3

- ME6211-3.3

- TLV75533

- XC6206P332

- HT7333

- Other 3.3 V LDO regulators

6. Replacing the ESD Protection Diode

Original Component

LESD5D5.0CT1G

Possible Replacements

- PESD5V series

- USBLC6 series

- TPD series USB protection devices

- Other low-capacitance TVS diodes

7. Replacing the Tactile Switch

Original Component

SKPMAME010

Possible Replacements

- SMD tactile switches from other manufacturers

- C&K tactile switches

- Omron tactile switches

- ALPS Alpine tactile switches

- Any standard 4-pin tactile switch with the same footprint

8. Replacing the LEDs

Original Components

- KT-0805G

- NCD0805R1

Possible Replacements

- 0805 LEDs from other manufacturers

- 0603 LEDs

- 1206 LEDs

- LEDs of different colors

9. Replacing Resistors

Original Package

0402

Compatible Package Options

- 0603

- 0805

- 1206

10. Replacing Capacitors

Items to Verify

- Capacitance

- Rated voltage

- Dielectric type

- Tolerance

- DC bias characteristics

- Equivalent Series Resistance (ESR)

- Package size

The effective capacitance of ceramic capacitors can decrease significantly as the applied DC voltage increases.

This effect is particularly noticeable in high-capacitance capacitors with small package sizes.

For example, a 10 μF capacitor in a 0402 package may provide considerably less than its nominal capacitance when operated at 3.3 V or 5 V.

If regulator stability or ESP32 power stability is important, consider using a 0603 or 0805 package instead.

11. Replacing the ESP32 Module

Original Component

ESP32-WROOM-32E

Possible Replacements

- ESP32-WROOM-32D

- ESP32-WROOM-32UE

- ESP32-WROVER series

- ESP32-S3-WROOM series

- ESP32-C3-WROOM series

Items to Verify

Even within the ESP32 family, different modules may vary in the following aspects:

- Module dimensions

- Number of pins

- Pin configuration

- Antenna type

- Flash memory capacity

- PSRAM availability

- Power requirements

- USB functionality

- Boot pins

- Available GPIO pins

- Input-only GPIO pins

- ADC characteristics

The ESP32-WROOM-32D and ESP32-WROOM-32E are largely compatible. However, migrating to an ESP32-S3 or ESP32-C3 typically requires significant modifications to both the hardware design and the firmware.

12. Replacing the Connectors

Original Component

Conn_01x17

Possible Replacements

- 2.54 mm Pin Header

- 2.00 mm Pin Header

- JST series connectors

- Board-to-Board connectors

- Screw terminals